signals

primitives

Top level ports

Jmp

module juanji(address,indata,outdata,wr,nconvst,nbusy,enout1,enout2,clk,reset,start);

input clk,						//采用10MHz的时钟

reset,					//复位信号

start,					//因为RAM的空间是有限的，当RAM存满后采样和卷积都会停止，此时给一个start的高电平脉冲将会开始下一次的卷积

nbusy;					//从A/D转换器来的信号表示转换器的忙或闲

output wr,						//RAM写控制信号

enout1,enout2,		//enout1是存储卷积低字节结果RAM的片选信号

//enout2是存储卷积高字节结果RAM的片选信号

nconvst,				//给出A/D转换器的控制信号，命令转换器开始工作，低电平有效

address;				//地址输出

input[7:0] indata;			//从A/D转换器来的数据总线

output[7:0] outdata;			//写到RAM去的数据总线

wire nbusy;

reg wr;

reg nconvst,enout1,enout2;

reg[7:0] outdata;

reg[10:0] address;

reg[8:0] state;

reg[15:0] result;

reg[23:0] line;

reg[11:0] counter;

reg high;

reg[4:0] j;

reg EOC;

parameter h1=1,h2=2,h3=3;						//假设的系统系数

parameter IDLE=9'b000000001,START=9'b000000010,NCONVST=9'b000000100,

READ=9'b000001000,CALCU=9'b000010000,WRREADY=9'b000100000,

WR  =9'b001000000,WREND=9'b010000000,WAITFOR=9'b100000000;

parameter FMAX=20;								//因为A/D转换的时间是随机的，为保证按一定的频率采样，

//A/D转换控制信号应以一定频率给出。这里的采样频率可通过FMAX控制，

//并设为500kHz

always@(posedge clk)

if(!reset)

begin

state		<= IDLE;

nconvst	<= 1'b1;

enout1	<= 1;

enout2	<= 1;

counter	<= 12'b0;

high		<= 0;

wr			<= 1;

line		<= 24'b0;

address	<= 11'b0;

end

else

case(state)

IDLE:

if(start==1)

begin

counter	<=0;					//counter是一个计数器，记录已用的RAM空间

line	 	<=24'b0;

state	 	<=START;

end

else

state			<=IDLE;

START:										//START状态控制A/D开始转换

if(EOC)

begin

nconvst	<=0;

high		<=0;

state		<=NCONVST;

end

else

state			<=START;

NCONVST:										//NCONVST状态是A/D转换保持阶段

begin

nconvst		<=1;

state			<=READ;

end

READ:											//READ状态读取A/D转换结果，计算卷积结果

begin

if(EOC)

begin

line	<={line[15:0],indata};

state	<=CALCU;

end

else

state		<=READ;

end

CALCU:

begin

result		<=line[7:0]*h1+line[15:8]*h2+line[23:16]*h3;

state			<=WRREADY;

end

//将卷积结果写入RAM时，先写入低字节，再写入高字节

WRREADY:										//WRREADY状态是写RAM准备状态，建立地址和数据信号

begin

address		<=counter;

if(!high)

outdata	<=result[7:0];

else

outdata	<=result[15:8];

state			<=WR;

end

WR:											//WR状态产生片选和写脉冲

begin

if(!high)

enout1	<=0;

else

enout2	<=0;

wr				<=0;

state			<=WREND;

end

WREND:										//WREND状态结束一次写操作,若还未写入高字节则转到WRREADY状态开始高字节写入

begin

wr				<=1;

enout1		<=1;

enout2		<=1;

if(!high)

begin

high	<=1;

state	<=WRREADY;

end

else

state		<=WAITFOR;

end

WAITFOR:										//WAITFOR状态控制采样频率并判断RAM是否已被写满

begin

if(j==FMAX-1)

begin

counter<=counter+1;

if(!counter[11])

state<=START;

else

begin

state<=IDLE;

$display($time,"The ram is used up.");

$stop;

end

end

else

state		<=WAITFOR;

end

default:	state		<= IDLE;

endcase

assign rd = 1;											//RAM的读信号始终保持高电平

//记录时钟，与FMAX共同控制采样频率，由于直接用clk的上升沿对nbusy判断，以决定某些操作是否运行时，

//会因为两个信号的跳变沿相隔太近而令状态机不能正常工作，因此利用clk的下降沿建立EOC信号与nbusy同步，

//相位相差180°，然后用clk的上升沿判断操作是否进行

always@(negedge clk)

begin

EOC <= nbusy;

if(!reset||state==START)

j<=1;

else

j<=j+1;

end

endmodule

//sram.v

module sram(Address,Data,SRG,SRE,SRW);

input[10:0] Address;

input SRG,								//Output enable

SRE,								//Chip enable

SRW;								//Write enable

inout[7:0] Data;						//Bus

wire[10:0] Addr=Address;

reg[7:0] RdData;

reg[7:0] SramMem[0:'h7ff];

reg RdSramDly,RdFlip;

wire[7:0] FlpData,Data;

reg WR_flag;							//To judge the signals according to the specification of HM-65162

integer i;

wire RdSram=~SRG&~SRE;

wire WrSram=~SRW&~SRE;

reg[10:0] DelayAddr;

reg[7:0] DelayData;

reg WrSramDly;

integer file;

assign FlpData = (RdFlip)?~RdData:RdData;

assign Data = (RdSramDly)?FlpData:'hz;

parameter TAVQV=90,					//2	(max)		Address access time

TELQV=90,					//3	(max)		Chip enable access time

TELQX=5,					//4	(min)		Chip enable output enable time

TGLQV=65,					//5	(max)		Output enable access time

TGLQX=5,					//6	(min)		Output enable output enable time

TEHQZ=50,					//7	(max)		Chip enable output disable time

TGHQZ=40,					//8	(max)		Output enable output disable time

TAVQX=5;					//9	(min)		Output hold from address change

parameter TAVWL=10,					//12	(min)		Address setup time

TWLWH=55,					//13	(min)		Chip enable pulse setup time

TWHAX=15,					//14	(min10)	Write enable read setup time

TWLQZ=50,					//16	(max)		Write enable output disable time

TDVWH=30,					//17	(min)		Data setup time

TWHDX=20,					//18	(min15)	Data hold time

TWHQX=20,					//19	(min0)	Write enable output enable time

TWLEH=55,					//20	(min)		Write enable pulse setup time

TDVEH=30,					//21	(min)		Chip enable data setup time

TAVWH=70;					//22	(min65)	Address valid to end of write

initial

begin

file=$fopen("ramlow.txt");

if(!file)

begin

$display("Could not open the file.");

$stop;

end

end

initial

begin

for(i=0;i<'h7ff;i=i+1)

SramMem[i]=i;

$monitor($time,"DelayAddr=%h,DelayData=%h",DelayAddr,DelayData);

end

initial RdSramDly=0;

initial WR_flag=1;

always@(posedge RdSram) #TGLQX RdSramDly=RdSram;

always@(posedge SRW)		#TWHQX RdSramDly=RdSram;

always@(Addr)

begin

#TAVQX;

RdFlip=1;

#(TGLQV-TAVQX)

if(RdSram)

RdFlip=0;

end

always@(posedge RdSram)

begin

RdFlip=1;

#TAVQV;

if(RdSram)

RdFlip=0;

end

always@(Addr)				#TAVQX RdFlip=1;

always@(posedge SRG)		#TEHQZ RdSramDly=RdSram;

always@(posedge SRE)		#TGHQZ RdSramDly=RdSram;

always@(negedge SRW)		#TWLQZ RdSramDly=0;

always@(negedge WrSramDly or posedge RdSramDly) RdData=SramMem[Addr];

always@(Addr)				#TAVWL DelayAddr=Addr;

always@(Data)				#TDVWH DelayData=Data;

always@(WrSram)			#5 	 WrSramDly=WrSram;

always@(Addr or Data or WrSram) WR_flag=1;

always@(negedge SRW)

begin

#TWLWH;

if(SRW)

begin

WR_flag=0;

$display("ERROR!Can't write!Write enable time(W) is too short!");

end

end

always@(posedge SRW)

begin

#TWHAX;

if(DelayAddr!==Addr)

begin

WR_flag=0;

$display("ERROR!Can't write!Write enable read setup time is too short!");

end

end

always@(Data)

if(WrSram)

begin

#TDVEH;

if(SRE)

begin

WR_flag=0;

$display("ERROR!Can't write!Chip enable data setup time is too short!");

end

end

always@(Data)

if(WrSram)

begin

#TDVEH;

if(SRW)

begin

WR_flag=0;

$display("ERROR!Can't write!Chip enable data setup time is too short!");

end

end

always@(posedge SRW)

begin

#TWHDX

if(DelayData!==Data)

$display("Warning!Data hold time is too short!");

end

always@(DelayAddr or DelayData or WrSramDly)

if(WrSram&&WR_flag)

begin

if(!Addr[5])

begin

#15 SramMem[Addr]=Data;

$display("mem[%h]=%h",Addr,Data);

$fwrite(file,"mem[%h]=%h",Addr,Data);

if(Addr[0]&&Addr[1])

$fwrite(file,"\n");

end

else

begin

$fclose(file);

$display("Please check the txt.");

$stop;

end

end

endmodule

module adc(nconvst,nbusy,data);

input nconvst;						//A/D启动脉冲ST

output nbusy;						//A/D工作标志

output data;						//数据总线，从AD.DATA文件中读取数据后经端口输出

reg[7:0] databuf,i;				//内部寄存器

reg nbusy;

wire[7:0] data;

reg[7:0] data_mem[0:255];

reg link_bus;

integer tconv,t5,t8,t9,t12;

integer wideth1,wideth2,wideth;

//时间参数定义

always@(negedge nconvst)

begin

tconv=9500+{$random}%500;				//(type 950,max 1000ns)Conversion Time

t5={$random}%1000;						//(max 100ns) CONVST to BUSY Propagation Dlay

//CL=10pf

t8=200;										//(min 20ns) CL=20pF Data Setup Time Prior to BUSY\

//(min 10ns) CL=100pF

t9=100+{$random}%900;					//(min 10ns,max 100ns) Bus Relinquish Time After CONVST

t12=2500;									//(type) BUSY High to CONVST Low, SHA Aquistion Time

end

initial

begin

$readmemh("adc.data",data_mem);		//从数据文件adc.data中读取数据

i=0;

nbusy=1;

link_bus=0;

end

assign data = link_bus?databuf:8'bzz;		//三态总线

always@(negedge nconvst)

fork

#t5 nbusy=0;

@(posedge nconvst)

begin

#tconv nbusy=1;

end

join

always@(negedge nconvst)

begin

@(posedge nconvst)

begin

#(tconv-t8) databuf=data_mem[i];

end

if(wideth<10000&&wideth>500)

begin

if(i==255)

i=0;

else

i=i+1;

end

else

i=i;

end

//在模数转换期间关闭三态输出，转换结束时启动三态输出

always@(negedge nconvst)

fork

#t9 link_bus=1'b0;						//关闭三态输出，不允许总线输出

@(posedge nconvst)

begin

#(tconv-t8) link_bus=1'b1;

end

join

//检查A/D启动信号的频率是否太快

always@(posedge nbusy)

begin

#t12;

if(!nconvst)

begin

$display("Warning!	SHA Acquisition Time is too short!");

end

else

$display("Warning!	SHA Acquisition Time is enough!");

end

//坚持A/D启动信号的负脉冲宽度是否足够和太宽

always@(negedge nconvst)

begin

wideth=$time;

@(posedge nconvst) wideth=$time-wideth;

if(wideth<=500||wideth>10000)

begin

$display("nCONVST Pulse Wideth=%d",wideth);

$display("Warning!	nCONVST Pulse Width is too narrow or too wide!");

$stop;

end

end

endmodule

module juanji(address,indata,outdata,wr,nconvst,nbusy,enout1,enout2,clk,reset,start);

input clk,						//采用10MHz的时钟

reset,					//复位信号

start,					//因为RAM的空间是有限的，当RAM存满后采样和卷积都会停止，此时给一个start的高电平脉冲将会开始下一次的卷积

nbusy;					//从A/D转换器来的信号表示转换器的忙或闲

output wr,						//RAM写控制信号

enout1,enout2,		//enout1是存储卷积低字节结果RAM的片选信号

//enout2是存储卷积高字节结果RAM的片选信号

nconvst,				//给出A/D转换器的控制信号，命令转换器开始工作，低电平有效

address;				//地址输出

input[7:0] indata;			//从A/D转换器来的数据总线

output[7:0] outdata;			//写到RAM去的数据总线

wire nbusy;

reg wr;

reg nconvst,enout1,enout2;

reg[7:0] outdata;

reg[10:0] address;

reg[8:0] state;

reg[15:0] result;

reg[23:0] line;

reg[11:0] counter;

reg high;

reg[4:0] j;

reg EOC;

parameter h1=1,h2=2,h3=3;						//假设的系统系数

parameter IDLE=9'b000000001,START=9'b000000010,NCONVST=9'b000000100,

READ=9'b000001000,CALCU=9'b000010000,WRREADY=9'b000100000,

WR  =9'b001000000,WREND=9'b010000000,WAITFOR=9'b100000000;

parameter FMAX=20;								//因为A/D转换的时间是随机的，为保证按一定的频率采样，

//A/D转换控制信号应以一定频率给出。这里的采样频率可通过FMAX控制，

//并设为500kHz

always@(posedge clk)

if(!reset)

begin

state		<= IDLE;

nconvst	<= 1'b1;

enout1	<= 1;

enout2	<= 1;

counter	<= 12'b0;

high		<= 0;

wr			<= 1;

line		<= 24'b0;

address	<= 11'b0;

end

else

case(state)

IDLE:

if(start==1)

begin

counter	<=0;					//counter是一个计数器，记录已用的RAM空间

line	 	<=24'b0;

state	 	<=START;

end

else

state			<=IDLE;

START:										//START状态控制A/D开始转换

if(EOC)

begin

nconvst	<=0;

high		<=0;

state		<=NCONVST;

end

else

state			<=START;

NCONVST:										//NCONVST状态是A/D转换保持阶段

begin

nconvst		<=1;

state			<=READ;

end

READ:											//READ状态读取A/D转换结果，计算卷积结果

begin

if(EOC)

begin

line	<={line[15:0],indata};

state	<=CALCU;

end

else

state		<=READ;

end

CALCU:

begin

result		<=line[7:0]*h1+line[15:8]*h2+line[23:16]*h3;

state			<=WRREADY;

end

//将卷积结果写入RAM时，先写入低字节，再写入高字节

WRREADY:										//WRREADY状态是写RAM准备状态，建立地址和数据信号

begin

address		<=counter;

if(!high)

outdata	<=result[7:0];

else

outdata	<=result[15:8];

state			<=WR;

end

WR:											//WR状态产生片选和写脉冲

begin

if(!high)

enout1	<=0;

else

enout2	<=0;

wr				<=0;

state			<=WREND;

end

WREND:										//WREND状态结束一次写操作,若还未写入高字节则转到WRREADY状态开始高字节写入

begin

wr				<=1;

enout1		<=1;

enout2		<=1;

if(!high)

begin

high	<=1;

state	<=WRREADY;

end

else

state		<=WAITFOR;

end

WAITFOR:										//WAITFOR状态控制采样频率并判断RAM是否已被写满

begin

if(j==FMAX-1)

begin

counter<=counter+1;

if(!counter[11])

state<=START;

else

begin

state<=IDLE;

$display($time,"The ram is used up.");

$stop;

end

end

else

state		<=WAITFOR;

end

default:	state		<= IDLE;

endcase

assign rd = 1;											//RAM的读信号始终保持高电平

//记录时钟，与FMAX共同控制采样频率，由于直接用clk的上升沿对nbusy判断，以决定某些操作是否运行时，

//会因为两个信号的跳变沿相隔太近而令状态机不能正常工作，因此利用clk的下降沿建立EOC信号与nbusy同步，

//相位相差180°，然后用clk的上升沿判断操作是否进行

always@(negedge clk)

begin

EOC <= nbusy;

if(!reset||state==START)

j<=1;

else

j<=j+1;

end

endmodule

//sram.v

module sram(Address,Data,SRG,SRE,SRW);

input[10:0] Address;

input SRG,								//Output enable

SRE,								//Chip enable

SRW;								//Write enable

inout[7:0] Data;						//Bus

wire[10:0] Addr=Address;

reg[7:0] RdData;

reg[7:0] SramMem[0:'h7ff];

reg RdSramDly,RdFlip;

wire[7:0] FlpData,Data;

reg WR_flag;							//To judge the signals according to the specification of HM-65162

integer i;

wire RdSram=~SRG&~SRE;

wire WrSram=~SRW&~SRE;

reg[10:0] DelayAddr;

reg[7:0] DelayData;

reg WrSramDly;

integer file;

assign FlpData = (RdFlip)?~RdData:RdData;

assign Data = (RdSramDly)?FlpData:'hz;

parameter TAVQV=90,					//2	(max)		Address access time

TELQV=90,					//3	(max)		Chip enable access time

TELQX=5,					//4	(min)		Chip enable output enable time

TGLQV=65,					//5	(max)		Output enable access time

TGLQX=5,					//6	(min)		Output enable output enable time

TEHQZ=50,					//7	(max)		Chip enable output disable time

TGHQZ=40,					//8	(max)		Output enable output disable time

TAVQX=5;					//9	(min)		Output hold from address change

parameter TAVWL=10,					//12	(min)		Address setup time

TWLWH=55,					//13	(min)		Chip enable pulse setup time

TWHAX=15,					//14	(min10)	Write enable read setup time

TWLQZ=50,					//16	(max)		Write enable output disable time

TDVWH=30,					//17	(min)		Data setup time

TWHDX=20,					//18	(min15)	Data hold time

TWHQX=20,					//19	(min0)	Write enable output enable time

TWLEH=55,					//20	(min)		Write enable pulse setup time

TDVEH=30,					//21	(min)		Chip enable data setup time

TAVWH=70;					//22	(min65)	Address valid to end of write

initial

begin

file=$fopen("ramlow.txt");

if(!file)

begin

$display("Could not open the file.");

$stop;

end

end

initial

begin

for(i=0;i<'h7ff;i=i+1)

SramMem[i]=i;

$monitor($time,"DelayAddr=%h,DelayData=%h",DelayAddr,DelayData);

end

initial RdSramDly=0;

initial WR_flag=1;

always@(posedge RdSram) #TGLQX RdSramDly=RdSram;

always@(posedge SRW)		#TWHQX RdSramDly=RdSram;

always@(Addr)

begin

#TAVQX;

RdFlip=1;

#(TGLQV-TAVQX)

if(RdSram)

RdFlip=0;

end

always@(posedge RdSram)

begin

RdFlip=1;

#TAVQV;

if(RdSram)

RdFlip=0;

end

always@(Addr)				#TAVQX RdFlip=1;

always@(posedge SRG)		#TEHQZ RdSramDly=RdSram;

always@(posedge SRE)		#TGHQZ RdSramDly=RdSram;

always@(negedge SRW)		#TWLQZ RdSramDly=0;

always@(negedge WrSramDly or posedge RdSramDly) RdData=SramMem[Addr];

always@(Addr)				#TAVWL DelayAddr=Addr;

always@(Data)				#TDVWH DelayData=Data;

always@(WrSram)			#5 	 WrSramDly=WrSram;

always@(Addr or Data or WrSram) WR_flag=1;

always@(negedge SRW)

begin

#TWLWH;

if(SRW)

begin

WR_flag=0;

$display("ERROR!Can't write!Write enable time(W) is too short!");

end

end

always@(posedge SRW)

begin

#TWHAX;

if(DelayAddr!==Addr)

begin

WR_flag=0;

$display("ERROR!Can't write!Write enable read setup time is too short!");

end

end

always@(Data)

if(WrSram)

begin

#TDVEH;

if(SRE)

begin

WR_flag=0;

$display("ERROR!Can't write!Chip enable data setup time is too short!");

end

end

always@(Data)

if(WrSram)

begin

#TDVEH;

if(SRW)

begin

WR_flag=0;

$display("ERROR!Can't write!Chip enable data setup time is too short!");

end

end

always@(posedge SRW)

begin

#TWHDX

if(DelayData!==Data)

$display("Warning!Data hold time is too short!");

end

always@(DelayAddr or DelayData or WrSramDly)

if(WrSram&&WR_flag)

begin

if(!Addr[5])

begin

#15 SramMem[Addr]=Data;

$display("mem[%h]=%h",Addr,Data);

$fwrite(file,"mem[%h]=%h",Addr,Data);

if(Addr[0]&&Addr[1])

$fwrite(file,"\n");

end

else

begin

$fclose(file);

$display("Please check the txt.");

$stop;

end

end

endmodule

`timescale 1ns / 100ps

////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date:   18:56:49 05/17/2023

// Design Name:   juanji

// Module Name:   D:/juan/juanjiqi.v

// Project Name:  juan

// Target Device:

// Tool versions:

// Description:

//

// Verilog Test Fixture created by ISE for module: juanji

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

////////////////////////////////////////////////////////////////////////////////

module juanjiqi;

wire wr,enin,enout1,enout2;

wire[10:0] address;

reg rd,clk,reset,start;

reg[7:0] indata;

wire nbusy,nconvst;

wire[7:0] outdata;

integer i;

parameter HALF_PERIOD=1000;

//产生10kHz的时钟

initial

begin

rd=1;

i=0;

clk=1;

forever #HALF_PERIOD clk = ~clk;

end

//产生置位信号

initial

begin

reset=1;

#(HALF_PERIOD*2+50) reset=0;

#(HALF_PERIOD*3) reset=1;

end

//产生开始卷积控制信号

initial

begin

start=0;

indata=8'b01011001;

#(HALF_PERIOD*7+20) start=1;

#(HALF_PERIOD*2) start=0;

#(HALF_PERIOD*1000) start=1;

#(HALF_PERIOD*2) start=0;

end

assign enin = 1;

juanji juanji(

.address(address),

.clk(clk),

.enout1(enout1),

.enout2(enout2),

.indata(indata),

.nbusy(nbusy),

.nconvst(nconvst),

.outdata(outdata),

.reset(reset),

.start(start),

.wr(wr)

);

sram ramlow(.Address(address),.Data(outdata),.SRW(wr),.SRG(rd),.SRE(enout1));

adc  adc(.nconvst(nconvst),.nbusy(nbusy),.data(indata));

endmodule